VANTAG RETAIL INTELLIGENCE PLATFORM

Edge Agent Installation Guide

Android / Windows / Pi / Jetson

| Document Code | VT-USR-019 |
| --- | --- |
| Category | User Guides |
| Version | 1.0 |
| Date | April 2026 |
| Status | Approved for Release |

© 2026 Vantag. Confidential. Not for redistribution.

Table of Contents

| 1 | Choosing a Device |
| --- | --- |
| 2 | Android |
| 3 | Windows |
| 4 | Raspberry Pi |
| 5 | NVIDIA Jetson |
| 6 | After Install |

1. Choosing a Device

| Device | Max Cameras | Cost | Best For |
| --- | --- | --- | --- |
| Old Android (8 core, 4 GB RAM) | 4 | ₹3,000 / S$49 / RM159 | Small shop |
| Windows mini-PC (Intel N100) | 10 | ₹14,000 / S$229 / RM699 | Mid shop |
| Raspberry Pi 5 (8 GB) | 6 | ₹9,500 / S$149 / RM479 | Shops, homes |
| NVIDIA Jetson Nano | 12 | ₹12,000 / S$199 / RM629 | Larger retail, mall zones |
| NVIDIA Jetson Orin Nano | 30 | ₹45,000 / S$699 / RM2299 | Malls, hospitals |

2. Android

Download Vantag Edge Agent APK from the 'Download Agent' button in dashboard.

Enable 'Install from unknown sources' for your browser once.

Open APK → install → launch.

Scan the QR code shown in dashboard.

Grant camera and storage permissions (only local — no cloud upload).

App will auto-start on reboot; pin to home for easy relaunch.

3. Windows

Diagram (powershell):

# Download installer from dashboard # Double-click vantag-agent-setup.exe # Paste pairing code when prompted # Installs as Windows Service, auto-starts on boot

4. Raspberry Pi

Diagram (bash):

# On Pi terminal curl -fsSL https://retail-vantag.com/install/pi.sh ¦ sudo bash sudo systemctl enable vantag-agent sudo systemctl start vantag-agent # Paste pairing code when prompted

5. NVIDIA Jetson

Diagram (bash):

sudo apt update && sudo apt install -y python3-pip ffmpeg pip3 install vantag-agent-jetson vantag-agent pair --code <PAIRING-CODE> # Model will auto-download and build TensorRT cache (first run ~4 min)

6. After Install

Dashboard shows Agent 'Online' within 60s

Run auto-discovery from dashboard → Cameras → Scan

Confirm discovered cameras (or add manually)

Start drawing zones